?.??	Indirect Domestic Hot Water Cylinder / Calorifier

The Mechanical Contractor shall supply and install an indirect domestic hot water cylinder as supplied by Commercial Hot Water Solutions Ltd (contact sales, email: sales@chwsltd.co.uk tel: 01934 244 770) from their ProStor SC range, in accordance with the following schedule:

- Model ref:				ProStor SC 500

- Vessel Capacity:			475 litres

- Coil Surface Area:			5 m2

The cylinder shall strictly comprise the following features; suitable for domestic hot water generation, high quality steel construction with WRAS approved vitreous lining, fixed single coil heat exchanger, 50mm hard polyurethane insulation with PVC outer jacket, 10 bar working pressure, 95 Deg C maximum working temperature, inspection hatch, destratification tappings, electrical immersion port and 3x ½” sensor tappings.