?.??	Indirect Domestic Hot Water Cylinder / Calorifier

The Mechanical Contractor shall supply and install an indirect domestic hot water cylinder as supplied by Commercial Hot Water Solutions Ltd (contact sales, email: sales@chwsltd.co.uk tel: 01934 244 770) from their ProStor SC range, in accordance with the following schedule:

- Model ref:				ProStor SC 1500

- Vessel Capacity:			1300 litres

- Coil Surface Area:			8 m2

The cylinder shall strictly comprise the following features; suitable for domestic hot water generation, high quality steel construction with WRAS approved vitreous lining, fixed single coil heat exchanger, 100mm soft polyurethane insulation with PVC outer jacket, 8 bar working pressure, 95 Deg C maximum working temperature, inspection hatch, destratification tappings, electrical immersion port and 3x ½” sensor tappings.