?.??	Domestic Hot Water Buffer Vessel

The Mechanical Contractor shall supply and install a domestic hot water buffer vessel as supplied by CHWS Ltd (contact sales, email: sales@chwsltd.co.uk tel: 01934 244 770) from their ProStor BV range, in accordance with the following schedule:

- Model ref:				ProStor BV 300

- Vessel Capacity:			285 litres

The cylinder shall strictly comprise the following features; suitable for domestic hot water storage, high quality steel construction, WRAS approved vitreous lining, Soft Polyurethane Insulation Jacket, 10 bar working pressure, 100 Deg C maximum working temperature, inspection hatch, destratification tappings, electrical immersion port and 3x ½” sensor tappings.