TECHSPEC_HC1000C_1223	Chilled Water Buffer Vessel

The Mechanical Contractor shall supply and install a chilled water buffer vessel, as supplied by Commercial Hot Water Solutions Ltd (contact CHWS, email: sales@chwsltd.co.uk tel: 01934 244770) from their ProStor HC range, in accordance with the following schedule:-

- Model ref:				ProStor HC 1000

- Vessel Capacity:			1000 litres

- Service Connections:		3”x 4 and 2”x 4

The ProStor HC Chilled Water Vessel shall strictly comprise the following features: high quality steel construction, Flexible Insulation with PVC Outer Jacket, electrical immersion port and 4x ½” sensor tapping’s for temperature monitoring. The unit shall have a maximum working pressure of 6 bar, test pressure of 9 bar and a maximum working temperature of 90 Deg C.